Chemical Reaction Engineering II- Reactor Design (ChEg 4101)

Prerequisite: Chemical Reaction Engineering I-Reaction Kinetics (ChEg3104)

Lecture 3: Design of Non-isothermal Ideal Batch Reactor

Week 3

Lecturer:  Adamu Esubalew (PhD, ChEg)

Lecture learning outcomes

At the end of the lecture, you will be able to:

- identify the characteristics of non-isothermal ideal batch reactor,

- determine change in heat of reaction at any temperature, and

- design non-isothermal ideal batch reactor using materials and energy balance equations.

3.1. Non-isothermal ideal batch reactor

Non-isothermal Batch reactor: It is a reactor in which reactants are mixed together and the reaction is allowed to proceed for a given time in a closed vessel with no inlet or outlet flows, and the reaction temperature varies with time. A non-isothermal ideal batch reactor is shown in Figure 3.1 (Fogler, 2015, pg. 632).  Energy is exchanged across the reactor boundary.

Figure 3.1. Non-isothermal batch reactor

For isothermal ideal batch reactor, the reaction time (tr) is determined using materials balance equation (See Lecture 2). i.e. no need of energy balance equation to find reaction time since reaction rate is independent of reaction temperature. The mixture volume may be variable or constant, which is dependent on the mixture's phases. The non-isothermal batch reactor considers the effect of temperature on the rate of reaction, conversion and concentration progresses for a given reaction time.

For non-isothermal batch reactor, both materials balance and energy balance equations   are mandatory to find reaction time (tr). Energy balance is necessary for non-isothermal reaction system to analyze the heat effect. The reaction rate is affected by reaction temperature. The conversion of reactants varies with reaction temperature. For non-isothermal batch reactor, concentrations, reaction rate and conversion are functions of reaction time and reaction temperature.

Non-isothermal ideal batch reactor can be operated either adiabatic or non-adiabatic condition. For adiabatic operation, there is no heat exchange between a reactor and a surrounding. i.e there is no heat addition into a reactor or heat removal from a reactor. However, for non-adiabatic operation, there is heat exchange between a reactor and a surrounding. i.e there is heat addition into a reactor or heat removal from a reactor.

2.2 Reaction time and auxiliary time

Reaction time (tr) is the time required of chemical reaction to reach a particular specified conversion.  It is determined by applying both materials balance and energy balance equations. Auxiliary time includes different periods of a batch operation such as raw material preparation time, discharging time and cleaning time. Total time is sum of reaction time and auxiliary time, which is the time elapsed to complete one batch operation.

2.3. The size or volume of batch reactor

The size or volume reactor required is the main design parameter for batch reactor. The size or volume of a batch reactor required is not directly calculated from material balance.  It is determined after calculating the cycle time (total time) required for a batch operation. The size or volume of a batch reactor is determined from the production rate (production capacity) and the amount of product formed per unit volume per total time.

V=PctcRv                                                         (3.1)

Where: V is volume of a batch reactor, Pc is production rate/ production capacity, Rv is amount of product formed per unit volume, and tc is total time/ cycle time.

2.4. Non-isothermal batch reactor design equations

Material balance equation

tr=t=NA00XAdXAV(-rA)                                                                             (3.2)

Where: t= tr is reaction time, NAo initial mole of species A, XA is conversion of species A, V is volume of mixture, and -rA is rate of reaction for species A.

The derivation details are found in isothermal batch reactor (Refer lecture 2). A simple batch reactor is illustrated in Figure 3.2 (Levenspiel, 2011, pg. 84).

.

Figure 3.2. Batch reactor

Energy balance equation

The general energy balance for unsteady state open system is given by:

Rate of accumulation = rate of input - rate of output + heat added ± work done

Q-WS+i=1mFiHi│in-i=1mFiHi│out=dEsysdt                                        (3.3)

No mass crosses the system boundaries since a batch reactor is closed system.

i=0mFi0Hi0=i=1mFiHi=0                                                                          (3.4)

The energy balance equation for a batch reactor is reduced to equation (3.5)

dEsydt=Q-WS                                                                                             (3.5)

Where: dEsysdt is the change in total energy, Q is addition of heat, t is reaction time, and Ws is work done by the system.

The total energy of the system is the sum of the products of specific energies (Ei) of various species in the system and the number of moles (Ni ) of that species.

The total energy of the system is given by:

Esys=i=1mNiEi                                                                                                  (3.6)

Ei=Ui+KEi+PEi                                                                                          (3.7)

For a batch reactor, kinetic and potential energies are neglected, hence equation (3.7) is reduced to equation (3.8):

Ei=Ui                                                                                                                 (3.8)

Where: Ui is specific internal energy.

Specific enthalpy is the sum of specific internal energy and specific pressure energy.

Hi= Ui+PVi                                                                                                               (3.9)

By re-arranging equation (3.9), it becomes:

Ui=Hi-PVi                                                                                                               (3.10)

By substituting   equation (3.10) into equation (3.8), you get

Ei=Hi-PVi                                                                                                            (3.11)

Where: Vi is molar specific volume, Hi is specific enthalpy, P is total pressure and Ui is specific internal energy.

By substituting equation (3.11) into equation (3.6), the total energy of the system is given by:

Esys=i=1mNiEi=i=1mNiHi-Pi=1mNiVi                                                   (3.12)

The term Pi=1mNiVi is equal to total pressure times total volume, PV

V=i=1mNiVi                                                                                                     (3.13)

Where: V is total volume of a mixture.

For constant volume, the PV term variation is practically smaller than the other terms, it can be neglected and equation (3.12) is simplified to equation (3.14).

Esys=i=1mNiHi                                                                                                (3.14)

By substituting   equation (3.14) in to equation (3.5), the energy balance equation of a batch reactor becomes:

Q-WS=d(i=1mNiHi)dt                                                                                       (3.15)

By differentiating the term i=1mNiHi, the energy balance of a batch reactor it becomes:

Q-WS=i=1mNidHidt+i=1mHidNidt                                                         (3.16)

dHidt  is rate of specific enthalpy of species.

dNidt  is rate of reaction of species.

Two criteria's to be considered in correlating Hi as function of T are when no phase changes involved and when phase changes involved.

Case 1: When no phase changes involved, enthalpy of specie, i at temperature of T is equal to enthalpy of formation of species, i at TR and the change in enthalpy in heating species i from TR to T.

Ethalpy of species i atT =Enthalpy offormation of species i atTR +∆HQ in heatingspecies i  fromTR to T

When no phase changes involved, Hi as function of T becomes:

Hi=HioTR+TRTCPidT                                                                          (3.17)

Where:Hi is specific enthalpy of species i at temperature T, Hoi is specific enthalpy of species i at TR,  CPi is specific heat capacity,  TR is reference temperature, 298K.

By differentiating  Hi with respect to time (equation 3.17), you get:

dHidt=CPidTdt                                                                                        (3.18)

Case 2: When phase changes involved from solid to gas, the enthalpy of species, i in the gas phase at T is equal to  the sum of enthalpy of formation of species, i in the solid phase at TR, enthalpy change in heating solid from TR to Tm,  heat of melting at Tm, enthalpy change in  heating liquid from Tm to Tb, heat of vaporization at Tb, and  enthalpy change in heating gas from Tb to T.

Ethalpy of species i in the gas phase at T=Enthalpy offormation of species i in the solid phaseat TR +∆HQ in heatingsolid fromTR to Tm+Heat ofmeltingat Tm +∆HQ in heatingliquid fromTm to Tb+Heat ofvaporizationat Tb +∆HQ in heatinggas fromTb to T

Where: Tm and Tb are melting and boiling temperatures, respectively.

When phase changes involved from a solid to a gas, then the enthalpy, Hi (T) of species of a gas at temperature T is given by:

Hi(T)=HioTR+TRTmCPsidT+∆HmiTm+TmTbCPlidT+∆HviTb+TbTCPgidT         (3.19)

By differentiating  Hi with respect to time, equation (3.19) becomes:

dHidt=CPsidTdt+CPlidTdt+CPgidTdt                                                                         (3.20)

Where: Cps, Cpl and Cpg are specific heat capacity of solid, liquid and gas phases, respectively.

Recalling the mole balance of species i in a batch reactor (see lecture 2).

dNidt=riV                                                                                                           (3.21)

Consider a reaction, A+baB→caC+daD                                                     (3.22)    dNidt=-νiriV                                                                                                               (3.23)

Where: ri is reaction rate of species i, vi is stochiometric coefficient of species i, A & B are reactants; C & D are products.

By substituting equations (3.18 and 3.23) into equation (3.16), the energy balance equation of a batch reactor becomes:

Q-WS=NiCPidTdt+νiHi (-riV)                                                              (3.24)

The summation of νiHi  for all species at temperature of T is the change in heat of reaction at T.

νiHi=∆HRx                                                                                             (3.25)

The stochiometric coefficient vi sign  is + ve for products and –ve for reactants.

To find the heat of reaction at a temperature of T, consider the stochiometric reaction as shown in equation (3.22).

A+baB→caC+daD

Recalling the specific enthalpy of species i at T for no phase change involved is given in equation (3.17).

Hi=H0iTR+TRTCPidT

For species i = A & i = B reactants, and i=C & i=D products, the specific enthalpy of species of each at T is given by:

HA=H0A(TR)+TRTCPAdT                                                                              (3.26)

HB=H0B(TR)+TRTCPBdT                                                                               (3.27)

HC=H0C(TR)+TRTCPCdT                                                                               (3.28)

HD=H0D(TR)+TRTCPDdT                                                                               (3.29)

Species A is considered as a limiting reactant. The change in heat of reaction at temperature T, per mole of A, is given by:

νiHi =∆HRxT=caHCT+daHDT-baHB(T)-HA(T)                 (3.30)

Substitute enthalpies of all species, equations (3.26- 3.29) into equation (3.30), you get:

νiHi =∆HRxT=caH0C+daH0D-baH0B-H0A+caTRTCPCdT+daTRTCPDdT-baTRTCPBdT-TRTCPAdT                                                                         (3.31)

The specific heat capacity of species, A, B, C & D is a function of temperature, T.

To determine the change in heat of reaction in equation (3.31), let’s consider two cases for specific heat capacity analysis. i.e. small and wide temperature ranges.

Case 1: For a small temperature range (between TR and T)

There is no any significant heat capacity variation of species i with temperature, T. So that   CPi is constant or the mean value is constant and take out from integral.

νiHi =caH0C+daH0D-baH0B-H0A+caCPC(T-TR)+daCPD(T-TR)-baCPB(T-TR)-CPA(T-TR    =caH0C+daH0D-baH0B-H0A+caCPC+daCPD-baCPB-CPA (T-TR)                                                                                         (3.32)

The first term of equation (3.32) is the change in heat of reaction of species at TR.

caH0C+daH0D-baH0B-H0A=∆H0Rx(TR)                                                       (3.33)

The second term of equation (3.32) is the change in specific heat capacity of species, at T.

caCPC+daCPD-baCPB-CPA=∆CP                                                                          (3.34)

νiHi =∆H0RxTR+∆CPT-TR=∆HRxT                                                    (3.35)

This is the change in heat of reaction at any temperature, T, for small temperature range.

Case II:  For a wide temperature range (between T and TR)

Heat capacities are strong functions of temperature.

Since heat capacities are variable, CPi cannot be taken out from integral, but the quadratic form of the heat capacity relation with temperature is approximated by:

CPi=αi+βiT+γiT2                                                                             (3.36)

For species i = A, the heat capacity and specific enthalpy are given by:

CPA=αA+βAT+γAT2                                                                            (3.37)

HA=H0A(TR)+TRTαA+βAT+γAT2dT                                             (3.38)

where, αA, βA, γA are temperature coefficients for species A.

For species i = B, the heat capacity and specific enthalpy are given by:

CPB=αB+βBT+γBT2                                                                            (3.39)

HB=H0B(TR)+TRTαB+βBT+γBT2dT                                             (3.40)

where, αB, βB, γB are temperature coefficients for species B.

For species i = C, the heat capacity and specific enthalpy are given by:

CPC=αC+βCT+γCT2                                                                             (3.41)

HC=H0C(TR)+TRTαC+βCT+γCT2dT                                             (3.42)

where, αC, βC, γC are temperature coefficients for species C.

For species i = D, the heat capacity and specific enthalpy are given by:

CPD=αD+βDT+γDT2                                                                             (3.43)

HD=H0D(TR)+TRTαD+βDT+γDT2dT                                             (3.44)

where, αD, βD, γD are temperature coefficients for species D.

Similarly, ∆HRxT for variable  CPi becomes:

HRx(T)=H0RX(TR)+TRTɑ+T+T2dT                               (3.45)

The integration of equation (3.45) becomes:

HRxT=H0RXTR+ɑT-TR+2(T2-T2R)+γ3(T3-T3R)  (3.46)

Where:

∆α=da D+ ca C- ba B- A

∆β=da D+ ca C- ba B- A                                                                     (4.47)

∆γ=da D+ ca C- ba B- A

The change in specific heat capacity at a temperature of T is equal to the difference between products and reactants at a temperature of T.

Hence, energy balance equation of a batch reactor can be simplified into equation (3.48).

dTdt=Q-WS+(-∆HRx)(-riV)NiCPi                                                                                       (3.48)

This is the general energy balance equation for non-isothermal ideal batch reactor.

After chemical reaction, the remaining number of moles for species, i on the basis of limiting reactant (species A) is given by:

Ni=Ni0+νiNA0XA =NA0Ni0NA0+νiXA=NA0(Θi+νiXA)                              (3.49)

Where: XA is conversion of species A, i is initial mole ratio of species, i to species A.

The temperature rate is simplified by substituting equation (3.49) into equation (3.48), and you get the following equations:

dTdt=Q-WS+(-∆HRx)(-rAV)NA0(Θi+νiXA) CPi                                                                       (3.50)

dTdt=Q-WS+(-∆HRx)(-rAV)NA0(ΘiCPi+νiCPiXA)                                                                     (3.51)

Where: νiCPi=∆CP

dTdt=Q-WS+(-∆HRx)(-rAV)NA0(ΘiCPi+∆CPXA)                                                                        (3.52)

Where: -rAV= dNA/dt= -NAodXA/dt

From materials balance equation (3.2) and energy balance equation (3.52), it is observed that conversion of reactant A is a function of reaction time and reaction temperature. The two equations are interdependent. Both equations must be solved simultaneously to find reaction time, temperature and conversion of reactants.

Design of non-isothermal ideal batch reactor with adiabatic operation

For adiabatic operation (Q = 0) and if the work done by stirrer is neglected, Ws≅0, the energy equation (3.52) becomes:

dTdt=-∆HRxNA0dXAdtNA0[ΘiCPi+∆CPXA]                                                                                                   (3.53)

By re-arranging equation (3.53), you get the following equations:

-∆HRxNA0dXAdt=NA0[ΘiCPi+∆CPXA]dTdt                                                                  (3.54)

-∆HRxdXA=[ΘiCPi+∆CPXA]dT                                                                             (3.55)

For small temperature range, ∆HRx=∆HRxo+∆CP(T-TR)                                       (3.35)

Substitute equation (3.35) into equation (3.55) , and solve for conversion (XA), you get:

XA=[ΘiCPi+∆CPXA](T-T0)-[∆HRxo+∆CPT-TR]                                                                                        (3.56)

If ∆CP is  =0,     XA=ΘiCPi-∆HRxo(T-T0)                                                                   (3.57)

This is a linear equation with slope: m=  ΘiCPi-∆HRxo  and intercept: b=  ΘiCPi-∆HRxoT0

XA=mT+b                                                                                                   (3.58)

Activity 3.1

A researcher wants to produce ethanol from glucose using fermentation process which is carried out in an adiabatic batch reactor. In the reaction, glucose (C6H12O6) is converted in to ethanol (C2H5OH) and carbon dioxide (CO2). The initial feed temperature is 25oC. The initial concentration of glucose is 6 mol/ L. The overall reaction can be represented as:

2C6H12O6 → 4C2H5OH + 4CO2.

2 A    4 B + 4 C

Rate Law: -rA = kCA2

Additional data are given below. Determine: (a) the temperature of the reactor when 80% of glucose is converted to ethanol, and(b) the concentration of produced ethanol.

Additional data

Standard heat of formations (H0i​) for each component at 298K: H0A (C6H12O6) = -1273 J/mol; H0B (C2H5OH) = -277 J/mol; H0C (CO2) = -394 J/mol

Specific Heat Capacities (CPi) at 298K: CPA(C6H12O6) = 13.2 J/mol·K; CPB(C2H5OH) = 4.88 J/mol·K; CPC(CO2) = 1.72 J/mol·K

The energy balance equation for non-isothermal ideal batch reactor is given by:

dTdt=Q+(-∆HRx)(-rAV)NA0(ΘiCPi+∆CPXA)                                                                                      (3.52)

Solution

The batch reactor operation is adiabatic, Q=0

Cp= 4CPC+4CPB-2CPA=4*(4.88+1.72)- 2*13.2= 0

∆HRx=∆HRxo+∆CP(T-TR)= ∆HRxo=4 *(HCo+ ∆HBo)-2* HAo

∆HRx=4*(-394-277)-2*(-1273)=-138J/mol

-∆HRxdXA=[ΘiCPi+∆CPXA]dT                                                                         (3.55)

-∆HRxdXA=[ΘiCPi]dT

ΘiCPi=ACPA+ BCPB+CCPC

ACPA=CPA=13.2 J/mol. K since A=NAoNAo=1,

B=NBoNAo=0 since NBo=0, C=NCoNAo=0 since NCo=0

-∆HRxXA=CPA(T-T0)

T=-∆HRxXACPA+ T0= (138*0.8) J/mol/(13.2 J/mol. K) +298 K= 306.36 K307 K

Design of non-isothermal   ideal batch reactor with non-adiabatic operation

For non- adiabatic operation, the energy balance equation (3.52) is simplified by substituting –rAV by –NAo dXA/dt as follows:

dTdt=Q  -Ws-∆HRxNA0dXAdtNA0[ΘiCPi+∆CPXA]                                                                           (3.59)

∆HRx=∆HRxo+∆CP(T-TR)

dTdt=Q  - Ws-[∆HRxo+∆CP(T-TR)]NA0dXAdtNA0[ΘiCPi+∆CPXA]                                                      (3.60)

For very small ∆CP, ∆CP≅0,  dTdt=Q  -Ws-∆HRxoNA0dXAdtNA0ΘiCPi                       (3.61)

If the work done by stirrer is not significant and neglected, Ws≅0, equation (3.61) becomes:

NA0ΘiCPidTdt=Q -∆HRxo NA0dXAdt                                                   (3.62)

The heat generation (Qg) and heat removal (Qr) can be accounted by.

Qg=-∆HRx-rAV                                                                               (3.63)

Qr=UA(Ta-T)                                                                                  (3.64)

Where         -Qr=Q

dTdt=Qg-Qr    NiCPi                                                                                            (3.65)

Where: NiCPi=NA0[ΘiCPi+∆CPXA]                                            (3.66)

For very small, ∆CP, ∆CP≅0, NiCPi=NA0ΘiCPi                        (3.67)

Energy balance for non-isothermal multiple reactions

dTdt=Q -WS + i=1qrijV∆HRx i j=1mNjCPj                                              (3.68)

Where:     q = number of multiple reactions

m = species occurring in batch reactor

Activity 3.2

A chemical engineer wants to produce ethanol from glucose using fermentation process which is carried out in an adiabatic batch reactor. In the reaction, glucose (C6H12O6) is converted in to ethanol (C2H5OH) and carbon dioxide (CO2). The overall reaction can be represented as:

C6H12O6 → 2C2H5OH + 2CO2

A    2 B + 2 C

Rate Law: r = k*CA2

Determine   the conversion, temperature and concentration profiles as a function of time when the reaction time is reached up to 90 min.

Design Data and information

Standard heat of formations (H0i​) for each component at 298K:

H0A (C6H12O6) = -1273 J/mol; H0B (C2H5OH) = -277 J/mol; H0C (CO2) = -394 J/mol

Specific Heat Capacities (CPi) at 298K:

CPi(C6H12O6) = 6.6 J/mol·K; CPi(C2H5OH) = 2.44 J/mol·K; CPi(CO2) = 0.86 J/mol·K

Initial feed temperature (T0) = 25°C; Rate constant at initial temperature (k at T0​) = 0.02 L/mol·min; activation energy (E) = 60 J/mol; Initial concentration of glucose = 1.5 mol/L; universal gas constant (R): 8.314 J/(mol·K); Molecular weight for ethanol= 46.08g/mol.

Solution

The energy balance equation for non-isothermal batch reactor is given by:

dTdt=Q+(-∆HRx)(-rAV)NA0(ΘiCPi+∆CPXA)                                                           (3.52)

Using the given data, simulate the conversion, concentrations (species A, B & C) and temperatures distributions as a function of time using polymath program. Demonstrate the plots and   discuss the simulation results.

- Conversion and reaction temperature profiles as a function of reaction time

Polymath program using polymath 6.10- trial version software tool:

d(XA) / d(t) = k*CA0* CA0 *(1-XA) *(1-XA) #Reaction of A-----------> 2B+2C

XA (0) = 0 # initial conversion

k=k0*exp(-E/(R*T)) # L/mol.min

k0=0.02 # L/ mol.min, E=60# J/mol, CA0=1.5 # mol/L, R=8.314 # J/ mol.K

d(T) / d(t) = (-HRx*(-rA))/(CA0*(1-XA)*CPA+ 2*CA0*XA*CPB+2*CA0*XA*CPC) #Adiabtic system

T(0) = 298 # K

rA=-k*CA0*CA0*(1-XA) *(1-XA) # mol/L.min

HRx= 2*(HC0+HB0)-HA0 # J/mol

HA0= -1273 # J/mol

HB0= -277 # J/mol

HC0= -394 # J/mol

CPA= 6.6#  J/ mol.K,

CPB= 2.44#  J/ mol.K,

CPC= 0.86#  J/ mol.K

t(0)=0 # min

t(f)= 90 # min

Simulation results

The simulation results are depicted in Figure 3.3 (a) conversion vs reaction time, (b) reaction temperature vs reaction time.

(a)

(b)

Figure 3.3 (a) conversion vs reaction time, and (b) reaction temperature vs reaction time

- Simulate concentrations of reactant A, product B & by-product C profiles as a function of reaction time.

Polymath program:

d(CA) / d(t) = -k*CA*CA # Reaction of A-----------> 2B+2C #  mol/ L. min

CA(0) = 1.5 # mol/L

d(CB) / d(t) = 2*k*CA*CA # mol/ L. min,

CB(0)=0  # mol/L

d(CC) / d(t) =2* k*CA*CA # mol/ L. min,

CC(0)=0  # mol/L

k=k0*exp(-E/(R*T)) # L/mol.min

k0=0.02 # L/ mol.min, E=60 # J/mol,

CA0=1.5 # mol/L,

R=8.314 # J/ mol.K

d(T) / d(t) = (-HRx*(-rA))/(CA*CPA+ CB*CPB+CC*CPC) #Adiabtic system

T(0) = 298 # K

rA= -k*CA*CA # mol/L.min,

HRx= 2*(HC0+HB0)-HA0 # J/mol

HA0= -1273 # J/mol,

HB0= -277 # J/mol,

HC0= -394 # J/mol,

CPA= 6.6#  J/ mol.K,

CPB= 2.44#  J/ mol.K,

CPC= 0.86#  J/ mol.K

t(0)=0 # min

t(f)= 90 # min

Simulation results: Simulation results are depicted in Figure 3.4 concentrations of reactant A, product B and by-product C vs reaction time.

Figure 3.4 Concentrations of reactant A, product B, and by-product C vs reaction time

Observations from simulation results

The fermentation process is an exothermic reaction. The reactant A (glucose) is converted into product B(ethanol) and by-product C (carbon dioxide). The conversion of reactant A is reached to 72% with reaction time of 90 min (Figure 3.3(a). The conversion of reactant A and reaction temperature are increased with reaction time (Figure 3.3). The concentration of the reactant A is decreased with reaction time because some portion of it is converted into product B and by-product C.  The concentrations of product B and by-product C are increased with reaction time, with the same quantity (Figure 3.4). The stoichiometric equation also confirms that the production of product B and by-product C are in equal number of moles. Product B and by-product C are plotted along the same line (Figure 3.4).

  - Summary

For non-isothermal batch reactor, concentrations, reaction rate and conversion are functions of reaction time and reaction temperature. Determining reaction temperature, conversion, reaction time and size or volume of reactor are main aims of non-isothermal batch reactor design. For non-isothermal operation, the change in heat of reaction at a temperature of T is sum the change in standard heat of reaction and the change in enthalpy from TR to T. For non-isothermal ideal batch reactor, both materials and energy balance equations must be solved simultaneously since they are mutually dependent.

References

- Chemical Reaction Engineering, Levenspiel, O John Wiely& Sons, 3rd edition, 2011.

- Elements of Chemical Engineering, Fogler, HS Prentice- Hall Inc Prentice- Hall Inc,5th edition, 2015.